# AWVT Audio Technician Grade 2 Standard Capture Template

Basic supported operation | Blank KSB and Assessment Criteria Working Document

Version 1.4 multi-source, venue-context draft populated working document

# Purpose of this grade document

This document is for the committee or subject specialists defining the AWVT Audio Technician Grade 2 standard. It should be populated with the Knowledge, Skills and Behaviours required at this grade, together with assessment methods and evidence requirements.

This is a blank working document. It does not yet define the final AWVT standard until completed, moderated and approved through the AWVT curriculum process.

# How the Grade Committee should use this document

This document is not intended to be completed by subject specialists working in isolation. The Grade Committee should use the draft content as a starting point, then review, amend, add, remove or move each KSB so that it sits at the correct grade. At least one educational professional should guide the process, checking that each KSB is written as a measurable learning outcome and that each assessment method produces reliable evidence at this grade only.

Draft content has been inserted from the earlier HoW Audio Technician occupational standard drafts. It should be treated as proposed material for committee review, not as an approved AWVT standard. This v1.3 version also adds KSB seed items drawn from Yamaha and selected microphone/RF resources, with links for committee review.

# Grade descriptor

| Item | Description |
| --- | --- |
| Grade descriptor | Routine assisted operation |
| Working competence statement | Candidate can carry out simple repeatable setup and operation tasks on an installed system with light supervision, including basic signal checking, gain awareness and safe start/end procedures. |
| Legacy source mapping | Earlier Level 1 material translated into the AWVT eight-grade structure. |
| Committee note | This draft content is a starting point. At least one educational professional should guide the committee to ensure outcomes, assessment methods and dependencies are written at the correct grade level. |

# Boundary with adjacent grades

| Boundary | Guidance |
| --- | --- |
| Lower boundary | Candidate should already hold or demonstrate the prerequisite KSBs listed in this document. |
| Upper boundary | Content requiring higher autonomy, complexity, leadership or technical judgement should be moved to the next grade. |
| Assessment boundary | Assessment must test competence at this grade only and must not depend on untaught KSBs. |

# Scope assumptions for this grade

The base audio system is assumed to be installed, connected and working. The learner may operate and adjust the system within the grade scope. Do not include tasks that require rigging, permanent installation, mains electrical work, work at height, structural work, or separate stage crew competence.

The committee should define any grade-specific limits below, such as equipment complexity, venue size, number of channels, number of performers, monitoring requirements, recording/streaming interface, or level of supervision.

| Scope area | Grade-specific limits to be defined |
| --- | --- |
| System starting point | The base audio system is already installed, connected, safe and working. |
| Excluded tasks | No learner is required to undertake rigging, stage-crew lifting, electrical installation or suspended-speaker work as part of this grade. |
| Permitted practical work | Learner may operate the system and add/adjust authorised items within local safety rules and grade competence. |
| Venue context | House of Worship and Amateur Arts Venue contexts, including services, rehearsals, performances and related events. |
| Assessment context | Assessment may be live, simulated or portfolio-based where the committee defines equivalent evidence. |

# KSB capture table

Use one row per KSB item. Add additional rows as required. Each KSB must be assessable and must identify any prior learning dependency.

| KSB ID | Type: K/S/B | Learner outcome statement | Scope / limits | Required prior learning | Assessment method | Evidence required | Pass standard | Merit / Distinction indicators |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2.K1 | K | Describe simple audio signal flow from input source through console channel to main output. | Installed system only; no complex routing. | A1.K1 | Oral questioning with system diagram | Annotated signal-flow sketch. | Shows correct input-to-output sequence. | Identifies likely break points in simple sequence. |
| A2.K2 | K | Know the difference between microphone, instrument/line-level source and DI connection at a practical introductory level. | Common HoW/AAV sources: speech mic, acoustic guitar DI, keyboard, playback device. | A1.K1 | Questioning and equipment matching | Matches source to usual connection method. | Correctly identifies common connection choices. | Explains why a DI may be used in simple terms. |
| A2.K3 | K | Understand basic gain/headroom awareness: too low, too high, clipping and audible distortion. | Console metering and listening only; no advanced metering. | A1.K2 | Practical demonstration and questioning | Candidate identifies safe/unsafe level examples. | Avoids clipping and recognises distortion. | Explains how gain and fader interact at a basic level. |
| A2.S1 | S | Carry out a simple channel line-check using an agreed sequence and report whether the source is present and routed. | Speech mic, DI or playback source; no complex routing changes. | A2.K1 | Observed practical line-check | Completed line-check notes. | Checks source, channel, mute and level in sequence. | Identifies whether problem is likely source, cable, channel or output path. |
| A2.S2 | S | Follow the local power-up and power-down sequence for the audio system. | Installed system only; no electrical work. | A1.K3; A2.K1 | Observed practical task | Assessor observes correct sequence. | Uses sequence that prevents loud pops and avoids equipment risk. | Explains why speakers/amps are normally last on and first off. |
| A2.S3 | S | Make basic level adjustments for speech and playback while maintaining a clear and comfortable sound. | One or two active sources at a time. | A1.S2; A2.K3 | Observed simulated service/event | Assessor listens and checks levels. | Maintains intelligibility without obvious distortion. | Responds smoothly to changes in speaker volume. |
| A2.S4 | S | Recognise likely feedback and take immediate basic corrective action within agreed limits. | Lower fader/mute and seek help; no EQ-based feedback control. | A2.K3 | Scenario-based practical task | Candidate responds to controlled feedback scenario. | Reduces feedback promptly and safely. | Identifies likely mic/speaker proximity cause. |
| A2.B1 | B | Follow written checklists and local procedures reliably. | Routine local procedures. | A1.B1 | Supervisor testimony / observation | Checklist completion and behaviour evidence. | Uses checklist without dismissing it as unnecessary. | Suggests sensible improvements through proper channels. |
| A2.K4 | K | Explain basic signal flow from a microphone or playback device to the main loudspeakers using the local mixer layout. | Known installed system only. | A1.K4 | Walk-through at the console | Candidate traces a nominated input to the main output without changing routing. | Identifies input channel, level control, output path and loudspeaker destination. | Also identifies where a monitor or recording/livestream feed may branch from the main path. Source reference: Yamaha Sound Reinforcement Handbook (Davis & Jones, 2nd ed.); Yamaha PA Beginner's Guide |
| A2.K5 | K | Recognise the meaning of common level indicators and basic dB/SPL language used by the audio team. | Recognition and safe operation only; no logarithm calculations. | A1.K2 | Oral questioning and meter observation | Candidate states what “too low”, “healthy”, and “too high/clipping” indications mean. | Keeps levels within the venue’s normal safe range and asks for help if clipping occurs. | Explains that dB is a relative scale and that small fader moves can be significant. Source reference: Yamaha Sound Reinforcement Handbook (Davis & Jones, 2nd ed.); Yamaha PA Beginner's Guide |
| A2.S4 | S | Set a basic starting level for one speech microphone using the venue checklist and assessor instruction. | One known wired or wireless speech microphone. | A1.S2; A2.K5 | Observed practical task | Assessor observes level setting against a known reference. | Achieves intelligible speech without obvious clipping or feedback. | Makes adjustments gradually while listening to the result. Source reference: Yamaha PA Beginner's Guide; Shure Microphone Basics |

# Assessment specification

Define how a candidate will be assessed at this grade. The assessment should be realistic, proportionate and capable of being repeated by different assessors with consistent results.

| Assessment element | Committee to define |
| --- | --- |
| Assessment scenario | Carry out a routine line-check and simple operation of speech/playback sources. |
| Minimum practical task | Follow power sequence, check one or more channels, avoid clipping and respond to basic feedback. |
| Knowledge questioning | Signal flow, DI/mic basics, gain/headroom and local safety procedure. |
| Evidence required | Observed practical task and completed checklist. |
| Pass standard | Completes routine task in sequence and reports issues correctly. |
| Merit indicator | Identifies likely location of simple faults. |
| Distinction indicator | Works smoothly with minimal prompting and useful self-checking. |

# Assessment criteria grid

| Criterion area | Pass | Merit | Distinction |
| --- | --- | --- | --- |
| Safety and procedure | Operates safely and follows local procedures. | Anticipates common risks and prevents avoidable issues. | Models safe practice and improves procedure where appropriate. |
| Technical accuracy | Completes required technical tasks correctly at the grade level. | Completes tasks reliably with limited prompting and good self-checking. | Shows assured control and explains decisions clearly. |
| Listening / communication quality | Achieves suitable clarity and audibility for the scenario. | Improves quality through appropriate adjustments. | Shows refined judgement appropriate to the grade and context. |
| Problem response | Recognises and reports/resolves problems within grade authority. | Identifies likely causes and takes proportionate action. | Recovers calmly and supports others while maintaining service continuity. |
| Team and behaviour | Works respectfully and reliably with others. | Communicates proactively and supports the event purpose. | Models mature, servant-hearted leadership and develops others. |

# Prerequisite and dependency mapping

Use this table to ensure that each KSB sits at the correct level. If the prerequisite does not exist in a lower grade or earlier in this grade, add it to the appropriate earlier point before approving this KSB.

| Current KSB ID | Depends on KSB ID | Dependency description | Where taught / assessed | Action needed |
| --- | --- | --- | --- | --- |
| A2.K1 | A1.K1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.K2 | A1.K1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.K3 | A1.K2 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.S1 | A2.K1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.S2 | A1.K3; A2.K1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.S3 | A1.S2; A2.K3 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.S4 | A2.K3 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A2.B1 | A1.B1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |

# Training source mapping

Use this table to record useful source material from training providers, manufacturers or existing courses. Source material may inform AWVT content, but it does not define the AWVT standard.

| Topic / KSB | Source organisation or course | Relevant material | Proposed use in AWVT grade | Reviewer notes |
| --- | --- | --- | --- | --- |
| Role context and duties | HoW Audio Technician Occupational Standard Overview | Occupation summary, duties, behaviour expectations and original competence-level outline. | Used as legacy source material to draft grade-appropriate KSBs. | Committee to verify language for HoW/AAV inclusivity. |
| Basic operation and safety | HoW Audio Technician L1 Occupational Standard | Routine tasks, safety, channel operation, signal checks and basic setup concepts. | Used mainly across Grades 1-3. | Confirm that rigging/stage crew tasks are excluded. |
| Intermediate operation | HoW Audio Technician L2 Occupational Standard | Mic choice, room acoustics, monitors, routing, emergency communication and collaboration. | Used mainly across Grades 3-6. | Split content into smaller grade steps. |
| Advanced operation | HoW Audio Technician L3 Occupational Standard | More autonomous operation, adaptation, system knowledge and fault handling. | Used mainly across Grades 5-8. | Committee should test against apprenticeship/outcome mapping. |
| Signal flow, levels and basic PA operation | Yamaha PA Beginner's Guide; Yamaha Sound Reinforcement Handbook | Signal path, meters, level, dB/SPL introductory language | Seed Grade 2 routine-operation KSBs | Avoid mathematics beyond recognition at this grade. |

# Moderation checklist

| Check | Confirmed / notes |
| --- | --- |
| Educational professional involved | Required or strongly recommended before approving final wording. |
| Each KSB has one assessable outcome | Check that outcomes avoid vague verbs such as understand unless paired with evidence. |
| No rigging/stage-crew tasks included | Confirm all tasks can be performed from an installed, working base system. |
| Prerequisites checked | Every dependency is either in a lower grade or earlier in this grade. |
| Assessment proportional | Assessment is realistic for volunteer learners and does not exceed the grade boundary. |
| ABRSM-style bands applied | Pass/Merit/Distinction are based on control, accuracy, confidence, communication and recovery. |
| HoW/AAV context inclusive | Language works for churches, other Houses of Worship and amateur arts venues. |
| Draft source verified | Committee confirms retained legacy material is accurate and current. |

# Document control

| Field | Entry |
| --- | --- |
| Document title | AWVT Audio Technician Grade 2 Standard Capture Template - Draft Populated Version |
| Version | 1.1 draft content inserted from earlier occupational standards |
| Status | Draft for committee review |
| Owner | AWVT Audio Grade Committee |
| Date | May 2026 |

# Source-informed draft additions and underlying prerequisite topics

Purpose. The following notes identify source-derived topics that appear to be assumed by later learning materials. Where a later-grade topic depends on a more fundamental idea, the prerequisite has been seeded at this grade or flagged for a lower grade. Committees should confirm the placement and wording before approval.

| Later topic / source assumption | Underlying prerequisite to place earlier | Suggested placement | Why it matters for assessment |
| --- | --- | --- | --- |
| Gain/headroom topics assume learners understand level indicators. | Safe meter/level awareness and basic dB/SPL vocabulary. | Grade 2 | Prevents clipping and excessive level before teaching gain structure. |
| Routing topics assume learners can trace a single input to output. | Basic local signal-flow walk-through. | Grade 2 | Allows later aux/group/routing learning to have a foundation. |

# Source link register for this grade

| Source | Link |
| --- | --- |
| Yamaha Sound Reinforcement Handbook (Davis & Jones, 2nd ed.) | Open source |
| Yamaha PA Beginner's Guide | Open source |
| Shure Microphone Basics | Open source |

# Venue-context role examples for this grade

This grade should be interpreted across Houses of Worship and other amateur-run venues. The examples below are not limiting; they help the Grade Committee keep the standard portable beyond a church-only context.

| AWVT Grade | Venue role description | Typical venue context | Trust / responsibility boundary |
| --- | --- | --- | --- |
| Grade 2 | Basic operator of a known system | Small church service, spoken-word event, rehearsal room, small meeting, simple acoustic act | Operate a familiar system using a checklist. Manage speech, playback and simple inputs without causing noise, feedback or interruption. |

# Multi-source topic validation for this grade

When confirming the KSBs in this document, the committee should check the proposed content against more than one source category where possible: core reference books, professional bodies/standards, manufacturer technical material and training-delivery resources. A YouTube video or manufacturer tutorial may explain a concept well, but the AWVT KSB should be written in non-brand-specific, assessable language.

| Topic area | Useful source categories | Committee check |
| --- | --- | --- |
| Device roles, safe operation, basic signal flow | Yamaha handbook/PA guides; Shure basics; Audio University | Check that the learner can recognise and operate only within a pre-configured system. |
| Venue conduct and safe assistance | Earlier HoW standards; local venue procedures | Assess behaviour and limits of responsibility, not just technical vocabulary. |

# Hidden prerequisite checks for this grade

For every topic inserted into this grade, the committee should identify whether the learner needs earlier knowledge before the KSB can be assessed fairly. If the prerequisite is missing, create or move a lower-grade KSB rather than forcing the learner to infer it.

| Candidate topic | Prior learning that may be required | Action for committee |
| --- | --- | --- |
| To be completed by committee | Identify any lower-grade knowledge, skill or behaviour assumed by this topic. | Add, move, split or rewrite the KSB so that the progression remains teachable. |
| To be completed by committee | Check whether the assessment assumes equipment or venue complexity beyond this grade. | Limit the scope or move to a higher grade. |