# AWVT Audio Technician Grade 4 Standard Capture Template

Developing control and judgement | Blank KSB and Assessment Criteria Working Document

Version 1.4 multi-source, venue-context draft populated working document

# Purpose of this grade document

This document is for the committee or subject specialists defining the AWVT Audio Technician Grade 4 standard. It should be populated with the Knowledge, Skills and Behaviours required at this grade, together with assessment methods and evidence requirements.

This is a blank working document. It does not yet define the final AWVT standard until completed, moderated and approved through the AWVT curriculum process.

# How the Grade Committee should use this document

This document is not intended to be completed by subject specialists working in isolation. The Grade Committee should use the draft content as a starting point, then review, amend, add, remove or move each KSB so that it sits at the correct grade. At least one educational professional should guide the process, checking that each KSB is written as a measurable learning outcome and that each assessment method produces reliable evidence at this grade only.

Draft content has been inserted from the earlier HoW Audio Technician occupational standard drafts. It should be treated as proposed material for committee review, not as an approved AWVT standard. This v1.3 version also adds KSB seed items drawn from Yamaha and selected microphone/RF resources, with links for committee review.

# Grade descriptor

| Item | Description |
| --- | --- |
| Grade descriptor | Secure local-system competence |
| Working competence statement | Candidate can manage a more complete local audio task, including basic monitor/foldback provision, radio microphone handling, fault quarantine and routine feedback prevention by positioning and level control. |
| Legacy source mapping | Earlier Level 1/2 material translated into the AWVT eight-grade structure. |
| Committee note | This draft content is a starting point. At least one educational professional should guide the committee to ensure outcomes, assessment methods and dependencies are written at the correct grade level. |

# Boundary with adjacent grades

| Boundary | Guidance |
| --- | --- |
| Lower boundary | Candidate should already hold or demonstrate the prerequisite KSBs listed in this document. |
| Upper boundary | Content requiring higher autonomy, complexity, leadership or technical judgement should be moved to the next grade. |
| Assessment boundary | Assessment must test competence at this grade only and must not depend on untaught KSBs. |

# Scope assumptions for this grade

The base audio system is assumed to be installed, connected and working. The learner may operate and adjust the system within the grade scope. Do not include tasks that require rigging, permanent installation, mains electrical work, work at height, structural work, or separate stage crew competence.

The committee should define any grade-specific limits below, such as equipment complexity, venue size, number of channels, number of performers, monitoring requirements, recording/streaming interface, or level of supervision.

| Scope area | Grade-specific limits to be defined |
| --- | --- |
| System starting point | The base audio system is already installed, connected, safe and working. |
| Excluded tasks | No learner is required to undertake rigging, stage-crew lifting, electrical installation or suspended-speaker work as part of this grade. |
| Permitted practical work | Learner may operate the system and add/adjust authorised items within local safety rules and grade competence. |
| Venue context | House of Worship and Amateur Arts Venue contexts, including services, rehearsals, performances and related events. |
| Assessment context | Assessment may be live, simulated or portfolio-based where the committee defines equivalent evidence. |

# KSB capture table

Use one row per KSB item. Add additional rows as required. Each KSB must be assessable and must identify any prior learning dependency.

| KSB ID | Type: K/S/B | Learner outcome statement | Scope / limits | Required prior learning | Assessment method | Evidence required | Pass standard | Merit / Distinction indicators |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A4.K1 | K | Understand feedback as a loop involving microphone, gain, speaker/monitor position and room response. | Placement/level solutions; EQ not yet primary method. | A3.K1; A2.K3 | Questioning with diagram/scenario | Candidate describes loop and basic controls. | Explains at least three practical ways to reduce feedback. | Prioritises non-destructive actions before EQ. |
| A4.K2 | K | Know the purpose and limitations of stage monitors, near-field monitors and in-ear monitoring at an introductory level. | Selection and operation within installed system. | A3.S1 | Oral questioning and system walk-through | Candidate identifies available monitor options. | Explains why performers need foldback and associated risks. | Links monitor choice to feedback and stage volume. |
| A4.K3 | K | Know local procedure for faulty equipment quarantine, cable labelling and repair/replacement escalation. | No mains electrical repair. | A1.K3; A2.S1 | Questioning and document review | Candidate explains quarantine process. | Separates faulty from usable equipment and records issue. | Provides useful fault description for repair decision. |
| A4.S1 | S | Set and adjust a simple foldback/monitor mix for one performer or group within system limits. | One aux/monitor send or equivalent; simple requests. | A4.K2; A3.S1 | Observed practical task | Performer/assessor feedback. | Provides usable foldback without creating obvious feedback risk. | Balances performer request against safe stage volume. |
| A4.S2 | S | Position microphones and speakers/monitors to improve coverage and reduce feedback risk. | No rigging; only repositioning items within authorised limits. | A4.K1; A3.K1 | Observed setup optimisation | Before/after assessor judgement. | Chooses safer, more effective positions within local constraints. | Explains trade-offs between coverage, sightlines and feedback. |
| A4.S3 | S | Operate and hand over a simple radio microphone system in accordance with local charging, muting and return procedure. | No RF planning; single or small number of known radio mics. | A3.K1 | Observed routine task | Checklist / assessor observation. | Checks battery/mute/status and recovers items after use. | Identifies when dropouts or interference need escalation. |
| A4.B1 | B | Take responsibility for routine audio quality while remaining within agreed authority. | Local events/services. | A3.B1 | Supervisor testimony / observation | Evidence from event support. | Acts reliably without overstepping competence or authority. | Keeps leaders informed of relevant limitations. |
| A4.K4 | K | Explain the difference between direct sound, reflected sound and reverberation in a HoW/AAV room. | Practical room-awareness level; no acoustic design calculations. | A3.K5 | Venue walk-round and oral questioning | Candidate identifies at least two room features affecting clarity. | Relates room acoustics to microphone placement, loudspeaker level and intelligibility. | Suggests a proportionate operational adjustment within the installed system limits. Source reference: Yamaha Sound Reinforcement Handbook (Davis & Jones, 2nd ed.); Shure Feedback: Fact and Fiction |
| A4.K5 | K | Know basic frequency-range language for speech, vocals and common HoW instruments, and how this informs EQ decisions. | Broad frequency bands only; no requirement for advanced critical listening. | A2.K5; A3.S4 | Questioning with practical examples | Candidate links low/mid/high changes to likely audible effects. | Uses EQ terminology appropriately and avoids excessive changes. | Can explain why boosting one source may mask another. Source reference: Yamaha Sound Reinforcement Handbook (Davis & Jones, 2nd ed.); Yamaha PA Beginner's Guide |
| A4.S4 | S | Position and operate a small number of microphones for speech, vocal or choir use while reducing avoidable spill and feedback risk. | No rigging or stand placement in unsafe areas; uses installed/available microphones only. | A3.K5; A4.K4 | Observed setup/rehearsal task | Assessor observes placement and listens to result. | Achieves usable pickup and respects sightlines, safety and worship/performance context. | Improves placement after listening and receiving performer feedback. Source reference: DPA Mic University; Shure Microphone Basics; Shure Microphone Directionality and Polar Pattern Basics |

# Assessment specification

Define how a candidate will be assessed at this grade. The assessment should be realistic, proportionate and capable of being repeated by different assessors with consistent results.

| Assessment element | Committee to define |
| --- | --- |
| Assessment scenario | Manage local-system operation with basic monitors/radio mics and routine fault procedures. |
| Minimum practical task | Set simple foldback, optimise mic/speaker placement, handle radio mic and quarantine a fault. |
| Knowledge questioning | Feedback loop, monitor purpose and local quarantine process. |
| Evidence required | Observed setup/event scenario and team feedback. |
| Pass standard | Operates safely and effectively within authority. |
| Merit indicator | Balances performer needs against feedback/stage-volume risk. |
| Distinction indicator | Explains practical trade-offs and prevents problems early. |

# Assessment criteria grid

| Criterion area | Pass | Merit | Distinction |
| --- | --- | --- | --- |
| Safety and procedure | Operates safely and follows local procedures. | Anticipates common risks and prevents avoidable issues. | Models safe practice and improves procedure where appropriate. |
| Technical accuracy | Completes required technical tasks correctly at the grade level. | Completes tasks reliably with limited prompting and good self-checking. | Shows assured control and explains decisions clearly. |
| Listening / communication quality | Achieves suitable clarity and audibility for the scenario. | Improves quality through appropriate adjustments. | Shows refined judgement appropriate to the grade and context. |
| Problem response | Recognises and reports/resolves problems within grade authority. | Identifies likely causes and takes proportionate action. | Recovers calmly and supports others while maintaining service continuity. |
| Team and behaviour | Works respectfully and reliably with others. | Communicates proactively and supports the event purpose. | Models mature, servant-hearted leadership and develops others. |

# Prerequisite and dependency mapping

Use this table to ensure that each KSB sits at the correct level. If the prerequisite does not exist in a lower grade or earlier in this grade, add it to the appropriate earlier point before approving this KSB.

| Current KSB ID | Depends on KSB ID | Dependency description | Where taught / assessed | Action needed |
| --- | --- | --- | --- | --- |
| A4.K1 | A3.K1; A2.K3 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A4.K2 | A3.S1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A4.K3 | A1.K3; A2.S1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A4.S1 | A4.K2; A3.S1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A4.S2 | A4.K1; A3.K1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A4.S3 | A3.K1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |
| A4.B1 | A3.B1 | Required prior learning before this KSB can be assessed. | Lower grade or earlier within this grade. | Confirm prior KSB exists and is assessed before this item. |

# Training source mapping

Use this table to record useful source material from training providers, manufacturers or existing courses. Source material may inform AWVT content, but it does not define the AWVT standard.

| Topic / KSB | Source organisation or course | Relevant material | Proposed use in AWVT grade | Reviewer notes |
| --- | --- | --- | --- | --- |
| Role context and duties | HoW Audio Technician Occupational Standard Overview | Occupation summary, duties, behaviour expectations and original competence-level outline. | Used as legacy source material to draft grade-appropriate KSBs. | Committee to verify language for HoW/AAV inclusivity. |
| Basic operation and safety | HoW Audio Technician L1 Occupational Standard | Routine tasks, safety, channel operation, signal checks and basic setup concepts. | Used mainly across Grades 1-3. | Confirm that rigging/stage crew tasks are excluded. |
| Intermediate operation | HoW Audio Technician L2 Occupational Standard | Mic choice, room acoustics, monitors, routing, emergency communication and collaboration. | Used mainly across Grades 3-6. | Split content into smaller grade steps. |
| Advanced operation | HoW Audio Technician L3 Occupational Standard | More autonomous operation, adaptation, system knowledge and fault handling. | Used mainly across Grades 5-8. | Committee should test against apprenticeship/outcome mapping. |
| Room acoustics, frequency language and practical mic placement | Yamaha Sound Reinforcement Handbook; DPA Mic University; Shure feedback/polar-pattern articles | Direct/reflected sound; reverberation; mic choice and placement; feedback reduction | Seed Grade 4 controlled mixing and problem-solving KSBs | Keep assessment practical and venue based. |

# Moderation checklist

| Check | Confirmed / notes |
| --- | --- |
| Educational professional involved | Required or strongly recommended before approving final wording. |
| Each KSB has one assessable outcome | Check that outcomes avoid vague verbs such as understand unless paired with evidence. |
| No rigging/stage-crew tasks included | Confirm all tasks can be performed from an installed, working base system. |
| Prerequisites checked | Every dependency is either in a lower grade or earlier in this grade. |
| Assessment proportional | Assessment is realistic for volunteer learners and does not exceed the grade boundary. |
| ABRSM-style bands applied | Pass/Merit/Distinction are based on control, accuracy, confidence, communication and recovery. |
| HoW/AAV context inclusive | Language works for churches, other Houses of Worship and amateur arts venues. |
| Draft source verified | Committee confirms retained legacy material is accurate and current. |

# Document control

| Field | Entry |
| --- | --- |
| Document title | AWVT Audio Technician Grade 4 Standard Capture Template - Draft Populated Version |
| Version | 1.1 draft content inserted from earlier occupational standards |
| Status | Draft for committee review |
| Owner | AWVT Audio Grade Committee |
| Date | May 2026 |

# Source-informed draft additions and underlying prerequisite topics

Purpose. The following notes identify source-derived topics that appear to be assumed by later learning materials. Where a later-grade topic depends on a more fundamental idea, the prerequisite has been seeded at this grade or flagged for a lower grade. Committees should confirm the placement and wording before approval.

| Later topic / source assumption | Underlying prerequisite to place earlier | Suggested placement | Why it matters for assessment |
| --- | --- | --- | --- |
| System EQ and room tuning assume room-acoustics vocabulary. | Direct/reflected sound and reverberation awareness. | Grade 4 | Lets learners distinguish source/mic problems from room problems. |
| Instrument mixing assumes broad frequency ranges. | Low/mid/high frequency language for common sources. | Grade 4 | Supports proportionate EQ decisions and reduces masking. |

# Source link register for this grade

| Source | Link |
| --- | --- |
| Yamaha Sound Reinforcement Handbook (Davis & Jones, 2nd ed.) | Open source |
| Shure Feedback: Fact and Fiction | Open source |
| Yamaha PA Beginner's Guide | Open source |
| DPA Mic University | Open source |
| Shure Microphone Basics | Open source |
| Shure Microphone Directionality and Polar Pattern Basics | Open source |

# Venue-context role examples for this grade

This grade should be interpreted across Houses of Worship and other amateur-run venues. The examples below are not limiting; they help the Grade Committee keep the standard portable beyond a church-only context.

| AWVT Grade | Venue role description | Typical venue context | Trust / responsibility boundary |
| --- | --- | --- | --- |
| Grade 4 | Developing live-sound operator | Worship group, amateur theatre rehearsal/performance, open mic, small band, community event | Mix several microphones or instruments, use basic EQ, understand feedback causes, manage simple monitor needs and adapt during a live event. |

# Multi-source topic validation for this grade

When confirming the KSBs in this document, the committee should check the proposed content against more than one source category where possible: core reference books, professional bodies/standards, manufacturer technical material and training-delivery resources. A YouTube video or manufacturer tutorial may explain a concept well, but the AWVT KSB should be written in non-brand-specific, assessable language.

| Topic area | Useful source categories | Committee check |
| --- | --- | --- |
| Gain structure, mic positioning, feedback, basic EQ | Yamaha; Shure; DPA Mic University; Audio University | Confirm prerequisites: signal flow, levels, mic/speaker roles and listening for change. |
| Routine live-event operation | Church Sound Training, Base Sound, QSC/HoW style training, practical venue procedures | Assess whether the learner can run a known event safely and calmly. |

# Hidden prerequisite checks for this grade

For every topic inserted into this grade, the committee should identify whether the learner needs earlier knowledge before the KSB can be assessed fairly. If the prerequisite is missing, create or move a lower-grade KSB rather than forcing the learner to infer it.

| Candidate topic | Prior learning that may be required | Action for committee |
| --- | --- | --- |
| To be completed by committee | Identify any lower-grade knowledge, skill or behaviour assumed by this topic. | Add, move, split or rewrite the KSB so that the progression remains teachable. |
| To be completed by committee | Check whether the assessment assumes equipment or venue complexity beyond this grade. | Limit the scope or move to a higher grade. |